|  | College of Engineering and Computer Science Mechanical Engineering Department Mechanical Engineering 375 Heat Transfer |
| --- | --- |
|  | Spring 2007 Number 17629 Instructor: Larry Caretto |

Solution to Fifth Quiz – Unsteady Heat Transfer

Stainless steel balls ( = 8085 kg/m3, k = 15.1 W/m·oC, and cp = 0.480 kJ/kg·oC,) with a diameter of 10 cm are to be quenched in water.  The balls have a uniform initial temperature of 900oC and are dumped into water at 30oC.  The heat transfer coefficient is 2500 W/m2·oC, Determine the time required for the center of the ball to reach a temperature of 50oC.  Use both the lumped parameter analysis and the charts and compare the results.  Which result is correct?

In the lumped parameter analysis, the time to reach a certain temperature is given by the following equation.

t=−1blnT−T∞Ti−T∞ b=hAρcpV=hρcpLc

From the data in the problem we can compute the parameter, b, and then compute the time for the temperature to reach 50oC.  For this ball which is a sphere of radius r = 5 cm = 0,05 m, the ratio of volume to area = (4r3/3)/(4r2) = r/3 = (.05 m)/3 = .01667 m.

b=hρcpLc=2500 Wm2⋅Cm38085 kgkg⋅C480 J10.01667 m1 JW⋅s=0.03865s

Here we have T = 30oC, T = 50oC, and Ti = 900oC Substituting these values and the value for b just found gives the following result for the time.

t=−1blnT−T∞Ti−T∞=−s0.03685ln50C−30C900C−30C=97.6 s

When using the charts to find the center temperature, we know the following parameters

T0−T∞Ti−T∞=50C−30C900C−30C=0.0230 khr=15.1 Wm⋅Cm2⋅C2500 W1.05 m=0.1208

Using these values in figure 4-17(a) gives the result that = t/r2 = 0.85 so that the time is

t=τr2α=ρcpτr2ktro2=m⋅C15.1 W8085 kgm3480 Jkg⋅C0.550.05 m2=353 s

This is a large difference!  Perhaps the lumped parameter approach is not valid.  To check this we compute the Biot number where the length parameter is the volume to area ratio, found to be 0.01667 m, above.

Bi=hLck=2500 Wm2⋅C0.01667 mm⋅C15.1 W=2.8

This Biot number is definitely not less than 0.1 so we know that the lumped parameter approach is not correct.  The correct answer is t = 353 s.