Chapter 6, Solution 80.

A geothermal power plant uses geothermal liquid water at 160ºC at a specified rate as the heat source. The actual and maximum possible thermal efficiencies and the rate of heat rejected from this power plant are to be determined.

Assumptions 1 The power plant operates steadily. 2 The kinetic and potential energy changes are zero. 3 Steam properties are used for geothermal water.

Properties Using saturated liquid properties, the source and the sink state enthalpies of geothermal water are (Table A-4)

Tsource=160 °C  hsource=675.47 kJ/kg Tsink=25 °C  hsink=104.83 kJ/kg

Analysis (a) The rate of heat input to the plant may be taken as the enthalpy difference between the source and the sink for the power plant

Qin=mgeo(hsource−hsink)=(440 kg/s)(675.47−104.83) kJ/kg=251,083 kW

The actual thermal efficiency is

ηth=Wnet,outQin=22 MW251.083 MW=0.0876=8.8%

(b) The maximum thermal efficiency is the thermal efficiency of a reversible heat engine operating between the source and sink temperatures

ηth,max=1−TLTH=1−(25+273) K(160+273) K=0.312=31.2%

(c) Finally, the rate of heat rejection is

Qout=Qin−Wnet,out=251.1−22=229.1 MW

Chapter 6, Solution 87.

The refrigerated space and the environment temperatures for a refrigerator and the rate of heat removal from the refrigerated space are given. The minimum power input required is to be determined.

Assumptions  The refrigerator operates steadily.

Analysis The power input to a refrigerator will be a minimum when the refrigerator operates in a reversible manner. The coefficient of performance of a reversible refrigerator depends on the temperature limits in the cycle only, and is determined from

-8C

25C

R

300 kJ/min

COPR, rev=1THTL−1=125+273  K−8+273  K−1=8.03

The power input to this refrigerator is determined from the definition of the coefficient of performance of a refrigerator,

Wnet, in, min=QLCOPR, max=300  kJ/min8.03=37.36  kJ/min=0.623 kW

Chapter 6, Solution 95.

A heat pump maintains a house at a specified temperature. The rate of heat loss of the house and the power consumption of the heat pump are given. It is to be determined if this heat pump can do the job.

Assumptions  The heat pump operates steadily.

Analysis The power input to a heat pump will be a minimum when the heat pump operates in a reversible manner. The coefficient of performance of a reversible heat pump depends on the temperature limits in the cycle only, and is determined from

5 kW

House

22C

HP

110,000 kJ/h

COPHP, rev=11−TLTH=11−2+273  K22+273  K=14.75

The required power input to this reversible heat pump is determined from the definition of the coefficient of performance to be

Wnet, in, min=QHCOPHP=110,000  kJ/h14.751  h3600  s=2.07  kW

This heat pump is powerful enough since 5 kW > 2.07 kW.

Chapter 6, Solution 97.

A heat pump maintains a house at a specified temperature in winter. The maximum COPs of the heat pump for different outdoor temperatures are to be determined.

Analysis The coefficient of performance of a heat pump will be a maximum when the heat pump operates in a reversible manner.  The coefficient of performance of a reversible heat pump depends on the temperature limits in the cycle only, and is determined for all three cases above to be

TL

20C

HP

COPHP, rev=11−TLTH=11−10+273 K20+273 K=29.3

COPHP, rev=11−TLTH=11−−5+273 K20+273 K=11.7

COPHP, rev=11−TLTH=11−−30+273 K20+273 K=5.86