ME470 Gas Mixtures HW Solutions          Inst:  Shoeleh Di Julio

Chapter 13, Solution 11.

The molar fractions of the constituents of a gas mixture are given. The gravimetric analysis of the mixture, its molar mass, and gas constant are to be determined.

Properties The molar masses of N2, and CO2 are 28.0 and 44.0 kg/kmol, respectively (Table A-1)

Analysis Consider 100 kmol of mixture.  Then the mass of each component and the total mass are

NN2=60 kmol  mN2=NN2MN2=60  kmol28  kg/kmol=1680  kgNCO2=40  kmol  mCO2=NCO2MCO2=40  kmol44  kg/kmol=1760  kg

mole

60% N2

40% CO2

mm=mN2+mCO2=1680  kg+1760  kg=3440  kg

Then the mass fraction of each component (gravimetric analysis) becomes

The molar mass and the gas constant of the mixture are determined from their definitions,

and

Chapter 13, Solution 33.

A tank contains a mixture of two gases of known masses at a specified pressure and temperature. The mixture is now heated to a specified temperature. The volume of the tank and the final pressure of the mixture are to be determined.

0.5 kmol Ar

2 kmol N2

280 K

250 kPa

Q

Assumptions Under specified conditions both Ar and N2 can be treated as ideal gases, and the mixture as an ideal gas mixture.

Analysis The total number of moles is

And     Nm=NAr+NN2=0.5 kmol+2 kmol=2.5 kmolVm=NmRuTmPm=(2.5 kmol)(8.314 kPa⋅m3/kmol⋅K)(280 K)250 kPa=23.3m3

Also,

P2V2T2=P1V1T1  P2=T2T1P1=400 K280 K(250 kPa)=357.1 kPa

Chapter 13, Solution 34.

The masses of the constituents of a gas mixture at a specified pressure and temperature are given. The partial pressure of each gas and the apparent molar mass of the gas mixture are to be determined.

Assumptions Under specified conditions both CO2 and CH4 can be treated as ideal gases, and the mixture as an ideal gas mixture.

Properties The molar masses of CO2 and CH4 are 44.0 and 16.0 kg/kmol, respectively (Table A-1)

Analysis The mole numbers of the constituents are

1 kg CO2

3 kg CH4

300 K

200 kPa

Then the partial pressures become

PCO2=yCO2Pm=0.108200  kPa=21.6  kPaPCH4=yCH4Pm=0.892200  kPa=178.4  kPa

The apparent molar mass of the mixture is

Chapter 13, Solution 37.

The volumetric fractions of the constituents of a gas mixture at a specified pressure and temperature are given. The mass fraction and partial pressure of each gas are to be determined.

Assumptions Under specified conditions all N2, O2 and CO2 can be treated as ideal gases, and the mixture as an ideal gas mixture.

Properties The molar masses of N2, O2 and CO2 are 28.0, 32.0, and 44.0 kg/kmol, respectively (Table A-1)

Analysis For convenience, consider 100 kmol of mixture. Then the mass of each component and the total mass are

65% N2

20% O2

15% CO2

350 K

300 kPa

NN2=65  kmol  mN2=NN2MN2=65  kmol28  kg/kmol=1820  kgO2O20  kmol

mm=mN2+mO2+mCO2=1820  kg+640  kg+660  kg=3120  kg

Then the mass fraction of each component (gravimetric analysis) becomes

mfN2=mN2mm=1820  kg3120  kg=0.583   or   58.3%mfO2=mO2mm=640  kg3120  kg=0.205   or   20.5%mfCO2=mCO2mm=660  kg3120  kg=0.212   or   21.2%

For ideal gases, the partial pressure is proportional to the mole fraction, and is determined from

PN2=yN2Pm=0.65300  kPa=195  kPaO20.20

Chapter 13, Solution 38.

The masses, temperatures, and pressures of two gases contained in two tanks connected to each other are given. The valve connecting the tanks is opened and the final temperature is measured. The volume of each tank and the final pressure are to be determined.

Assumptions Under specified conditions both N2 and O2 can be treated as ideal gases, and the mixture as an ideal gas mixture

Properties The molar masses of N2 and O2 are 28.0 and 32.0 kg/kmol, respectively (Table A-1)

# Analysis The volumes of the tanks are

1 kg N2

25C

300 kPa

3 kg O2

25C

500 kPa

VN2=mRTPN2=(1  kg)(0.2968  kPa⋅m3/kg⋅K)(298  K)300  kPa=0.295  m3VO2=mRTPO2=(3  kg)(0.2598  kPa⋅m3/kg⋅K)(298  K)500  kPa=0.465  m3

Vtotal=VN2+VO2=0.295 m3+0.465  m3=0.76  m3

Also,

mN2=1 kg  NN2=mN2MN2=1  kg28  kg/kmol=0.03571  kmolmO2=3  kg  NO2=mO2MO2=3  kg32  kg/kmol=0.09375  kmol

Nm=NN2+NO2=0.03571 kmol+0.09375  kmol=0.1295  kmol

Thus,

Pm=NRuTVm=(0.1295  kmol)(8.314  kPa⋅m3/kmol⋅K)(298  K)0.76  m3=422.2   kPa

Chapter 13, Solution 39.

The volumes, temperatures, and pressures of two gases forming a mixture are given. The volume of the mixture is to be determined using three methods.

# Analysis (a) Under specified conditions both O2 and N2 will considerably deviate from the ideal gas behavior. Treating the mixture as an ideal gas,

N2 + O2

200 K

8 MPa

0.3 m3 O2

200 K

8 MPa

0.5 m3 N2

200 K

8 MPa

NO2=PVRuTO2=(8000  kPa)(0.3  m3)(8.314  kPa⋅m3/kmol⋅K)(200  K)=1.443  kmolNN2=PVRuTN2=(8000  kPa)(0.5  m3)(8.314  kPa⋅m3/kmol⋅K)(200  K)=2.406  kmol

Nm=NO2+NN2=1.443  kmol+2.406  kmol=3.849  kmolVm=NmRuTmPm=(3.849  kmol)(8.314  kPa⋅m3/kmol⋅K)(200  K)8000  kPa=0.8 m3

(b)  To use Kay's rule, we need to determine the pseudo-critical temperature and pseudo-critical pressure of the mixture using the critical point properties of O2 and N2 from Table A-1.  But we first need to determine the Z and the mole numbers of each component at the mixture temperature and pressure (Fig. A-15),

O2:	TR, O2=TmTcr, O2=200  K154.8  K=1.292 ZO2=0.77

N2:	TR, N2=TmTcr, N2=200 K126.2 K=1.585 ZN2=0.863

NO2=PVZRuTO2=(8000  kPa)(0.3  m3)(0.77)(8.314 kPa⋅m3/kmol⋅K)(200  K)=1.874  kmolNN2=PVZRuTN2=(8000  kPa)(0.5  m3)(0.863)(8.314  kPa⋅m3/kmol⋅K)(200  K)=2.787  kmol

Nm=NO2+NN2=1.874  kmol+2.787  kmol=4.661  kmol

The mole fractions are

yO2=NO2Nm=1.874 kmol4.661 kmol=0.402 yN2=NN2Nm=2.787 kmol4.661 kmol=0.598Tcr, m'=yiTcr, i=yO2Tcr, O2+yN2Tcr, N2

Then,

TR=TmTcr, O2'=200  K137.7  K=1.452 Zm=0.82	(Fig. A-15)

Thus,	Vm=ZmNmRuTmPm=(0.82)(4.661  kmol)(8.314  kPa⋅m3/kmol⋅K)(200  K)8000  kPa=0.79 m3

(c)  To use the Amagat’s law for this real gas mixture, we first need the Z of each component at the mixture temperature and pressure, which are determined in part (b).  Then,

Zm=yiZi=yO2ZO2+yN2ZN2=0.4020.77+0.5980.863=0.83

Thus, 	Vm=ZmNmRuTmPm=(0.83)(4.661  kmol)(8.314  kPa⋅m3/kmol⋅K)(200  K)8000  kPa=0.80 m3

Chapter 13, Solution 55.

The temperatures and pressures of two gases forming a mixture in a mixing chamber are given. The mixture temperature and the rate of entropy generation are to be determined.

Assumptions 1 Under specified conditions both C2H6 and CH4 can be treated as ideal gases, and the mixture as an ideal gas mixture. 2 The mixing chamber is insulated and thus there is no heat transfer. 3 There are no other forms of work involved. 3 This is a steady-flow process. 4 The kinetic and potential energy changes are negligible.

20C

9 kg/s

C2H6

45C

4.5 kg/s

CH4

200 kPa

Properties The specific heats of C2H6 and CH4 are 1.7662 kJ/kg.C and 2.2537 kJ/kg.C, respectively. (Table A-2b).

Analysis (a) The enthalpy of ideal gases is independent of pressure, and thus the two gases can be treated independently even after mixing. Noting that , the steady-flow energy balance equation reduces to

Ein−Eout=ΔEsystemÃ0(steady)=0Ein=Eoutmihi=mehe0=mehe−mihi=mC2H6he−hiC2H6+mCH4he−hiCH40=mcpTe−TiC2H6+mcpTe−TiCH4

Using cp values at room temperature and substituting, the exit temperature of the mixture becomes

0=9  kg/s1.7662  kJ/kg⋅°CTm−20°C+4.5  kg/s2.2537  kJ/kg⋅°CTm−45°CTm=29.7°C 302.7  K

(b)  The rate of entropy change associated with this process is determined from an entropy balance on the mixing chamber,

Sin−Sout+Sgen=ΔSsystemÃ0=0[m(s1−s2)]C2H6+[m(s1−s2)]CH4+Sgen=0  →   {Sgen=[m(s2−s1)]C2H6+[m(s2−s1)]CH4

The molar flow rate of the two gases in the mixture is

NC2H6=mMC2H6=9  kg/s30  kg/kmol=0.3  kmol/s              {NCH4=mMCH4=4.5  kg/s16  kg/kmol=0.2813  kmol/s

Then the mole fraction of each gas becomes

yC2H6=0.30.3+0.2813=0.516             yCH4=0.28130.3+0.2813=0.484

Thus,

(s2−s1)C2H6=cplnT2T1−Rlny Pm, 2P1C2H6=cplnT2T1−RlnyC2H6=(1.7662  kJ/kg⋅K) ln302.7  K293  K−(0.2765  kJ/kg⋅K) ln(0.516)=0.240  kJ/kg⋅K(s2−s1)CH4=cplnT2T1−Rlny Pm, 2P1CH4=cplnT2T1−RlnyCH4

Noting that   and substituting,

Sgen=9  kg/s0.240  kJ/kg⋅K+4.5  kg/s0.265  kJ/kg⋅K=3.353 kW/K

Chapter 13, Solution 63.

Heat is transferred to a gas mixture contained in a piston cylinder device. The initial state and the final temperature are given. The heat transfer is to be determined for the ideal gas and non-ideal gas cases.

Properties The molar masses of H2 and N2 are 2.0, and 28.0 kg/kmol. (Table A-1).

Analysis  From the energy balance relation,

6 kg H2

21 kg N2

5 MPa

160 K

Q

Ein−Eout=ΔEQin−Wb,out=ΔUQin=ΔH=ΔHH2+ΔHN2=NH2h2−h1H2+NN2h2−h1N2

since Wb and U combine into H for quasi-equilibrium constant pressure processes

(a)  Assuming ideal gas behavior, the inlet and exit enthalpies of H2 and N2 are determined from the ideal gas tables to be

Thus, 	Qideal=3×5,669.2−4,535.4+0.75×5,810−4,648=4273   kJ

(b)  Using Amagat's law and the generalized enthalpy departure chart, the enthalpy change of each gas is determined to be

H2:	TR1, H2=Tm, 1Tcr, H2=16033.3=4.805PR1, H2=PR2, H2=PmPcr, H2=51.30=3.846  Zh10Zh20		(Fig. A-29)

Thus H2 can be treated as an ideal gas during this process.

N2:	TR1, N2=Tm, 1Tcr, N2=160126.2=1.27PR1, N2=PR2, N2=PmPcr, N2=53.39=1.47  Zh1=1.3Zh2=0.7		(Fig. A-29)

Therefore,

h2−h1H2=h2−h1H2, ideal=5,669.2−4,535.4=1,133.8 kJ/kmolh2−h1N2=RuTcrZh1−Zh2+h2−h1ideal

Substituting,

Qin=3  kmol1,133.8  kJ/kmol+0.75  kmol1,791.5  kJ/kmol=4745 kJ

Chapter 13, Solution 65.

Air is compressed isothermally in a steady-flow device. The power input to the compressor and the rate of heat rejection are to be determined for ideal  and non-ideal gas cases.

Assumptions 1 This is a steady-flow process. 2 The kinetic and potential energy changes are negligible.

Properties The molar mass of air is 29.0 kg/kmol. (Table A-1).

200 K

4 MPa

79% N2

21% O2

200 K

8 MPa

Analysis The mass flow rate of air can be expressed in terms of the mole numbers as

(a)  Assuming ideal gas behavior, the h and s of air during this process is

Δh=0 isothermal processΔs=cplnT2T1Ã0−RulnP2P1=−RulnP2P1−8.314  kJ/kg⋅K ln8  MPa4  MPa=−5.763  kJ/kmol⋅K

Disregarding any changes in kinetic and potential energies, the steady-flow energy balance equation for the isothermal process of an ideal gas reduces to

Also for an isothermal, internally reversible process the heat transfer is related to the entropy change by

,

Q=NTΔs=0.10  kmol/s200  K−5.763  kJ/kmol⋅K=−115.3 kW   →    {Qout=115.3 kW

Therefore,

Win=Qout=115.3  kW

(b) Using Amagat's law and the generalized charts, the enthalpy and entropy changes of each gas are determined from

where

N2:	R1Pm, 1Pcr, N243.39TR1=TR2=TmTcr, N2=220126.2=1.74 R2Pm, 2Pcr, N283.39	(Tables A-29 and A-30)

O2:	R1Pm, 1Pcr, O245.08TR1=TR2=TmTcr, O2=220154.8=1.421 R2Pm, 2Pcr, O285.08	(Tables A-29 and A-30)

Then,

h2−h1=yiΔhi=yN2(h2−h1)N2+yO2(h2−h1)O2

Thus,

Qout=−NTΔs=−0.10  kmol/s200  K−7.18  kJ/kmol⋅K=143.6   kWEin−Eout=ΔEsystemÃ0(steady)=0Ein=EoutWin+Nh1=Qout+Nh2Win=Qout+N(h2−h1)  Win=143.6 kW+(0.10 kmol/s)(−494 kJ/kmol)=94.2  kW